# Cryptographically Secure Random Number Generator utilising Environmental Radiation

Pan Ziyue
Engineering Product Development
Singapore University of Technology & Design
ziyue_pan@mymail.sutd.edu.sg

Chua Kevin Subong
Engineering Product Development
Singapore University of Technology & Design
kevin_chua@mymail.sutd.edu.sg

Ng Au Hern Wesley
Engineering Product Development
Singapore University of Technology & Design
wesley_ng@mymail.sutd.edu.sg

Kaung Khant Htet
Engineering Product Developmen
Singapore University of Technology & Design
khanthtet_kaung@mymail.sutd.edu.sg
 
Shum Hei Lam
Engineering Product Development
Singapore University of Technology & Design
heilam_shum@mymail.sutd.edu.sg

Diagram of our proposed design

Abstract — Generating random numbers that are cryptographically secure can be expensive with dedicated hardware. This paper describes the process of creating a cost-effective Cryptographically Secure Random Number Generator on an FPGA coupled to a Geiger counter. The frontend consists of a Geiger counter system that outputs pulses in response to environmental radioactivity, as well as a neoTRNG-based ring oscillator RNG. The backend uses a BLAKE2s hash function to remove any potential biasness created by the frontend. The paper will provide an insight into the conceptualization and implementation processes when creating the CSRNG, as well as highlighting challenges faced.

# Introduction

From computing to cryptography, random numbers are essential for various algorithms used in our computer systems. However, computers, being deterministic machines, can never generate truly random numbers on their own. Instead, they rely on Pseudo-random Number Generators (PRNGs), algorithms designed to produce sequences of numbers that each simulate the properties of a random number.

PRNGs use a seed value to generate a sequence of numbers through mathematical formulas or state machines which can appear randomly but are entirely determined by the seed value. However, this makes PRNGs highly predictable, which can make them unviable for cryptographic uses as they may be vulnerable to cyber-attacks. As such, creating a True Random Number Generator (TRNG) is key to ensuring information security.

TRNGs, unlike PRNGs, derive their randomness directly from entropy sources, which are inherently unpredictable processes, such as electronic noise and radioactive decay. By seeding PRNGs through such unpredictable phenomena, non-deterministic outputs can be output, making them highly secure for generating encryption keys and truly random    numbers for cryptographic algorithms. The paper focuses on an implementation of such an RNG that utilizes radioactive decay detected in the environment, as well as ring oscillator jitter noise as input data to be used as a cryptographically secure random number generator (CSRNG).

# Background and Motivation

Ensuring true randomness is especially critical in cryptography, where deterministic patterns may compromise key generation and encryption schemes. Recent works have demonstrated the viability of hybrid RNG systems utilizing physical entropy sources. Inspired by radiation-based RNG systems such as the one in Bonacini's implementation [1], and supplemented by techniques described in quantum RNG literature [2], this project aims to combine high quality physical entropy with cryptographically secure post-processing, limiting the possibilities of side-channel attacks.

# Frontend Circuitry

## Geiger Counter

Geiger counter GQ GMC-1130 used as radiation detection

To detect and retrieve information regarding radioactive decay in the environment, a Geiger counter was used. Geiger counter model GQ GMC-1130 was used to accurately retrieve this information. The counter has a 3.5mm analog data output port and a USB port, where the former is used to retrieve analog signals used as a data input.

## Radioactive Source

Americium-241 ion chamber used as radioactive source

A smoke detector ion chamber containing Americium-241 was originally to be used as the radioactive source. However, due to the shielding around the Americium for safety reasons, there is a negligible change in readings on the counter. Throughout this paper, the Americium was not removed from this shielding, to minimize harm towards people in the environment. As such, background radioactivity is used for detection instead. Future implementations could include a properly designed module which hermetically seals the Americium while allowing alpha particles to pass through (e.g. mica window) to the Geiger counter.

## Schmitt trigger for Geiger counter data input

The Geiger counter outputs a pulse signal which must be converted to a digital signal to be read by the Field-Programmable Gate Array (FPGA). To do this, a circuit consisting of resistors and op amps was used, forming a Schmitt Trigger that converts the pulse signal of the Geiger counter into square waves that adhere to the FPGA’s static discipline.

Analog output direct from Geiger counter

Non-Inverting Gain and Comparator amplifier circuitry

Physical circuitry with Geiger counter and FPGA

After the Schmitt Trigger, the signal adheres outputs a low value of 0, and a high value of 2.26V, which adheres to the VIH and VIL of the FPGA I/O pins. VIH is the voltage at which an input voltage above this value is recognized as high, while VIL is the voltage at which an input voltage below this value is recognised as low.

## Digitalization through fast clock method

To digitize the signal, the fast clock method is used. This approach converts the temporal spacing between radiation events into 8-bit numerical values

Fast clock method: A fast clock (down) is used to increase a counter. Whenever a detection is made (up), the counter is read and reset, generating one random number. Image retrieved from [1].

This method was chosen as opposed to the slow clock method and time difference method due to the low number of counts generated by environmental radioactivity. The fast clock method will be able to generate a higher deviation of numbers that are non-zero compared to the slow clock method, and the generation of bits will also be faster compared to the time difference method.

Testbench of fastclock module in Vivado. Gpio reflectes incoming rising edges from the Geiger counter circuit.

## Additional Entropy Source

- To increase available entropy of the seeding system, a ring oscillator jitter-based RNG, based off the neoTRNG module, is implemented. A ring oscillator is a chain of inverters connected in a loop, forming a feedback circuit that toggles rapidly due to propagation delays. By extracting timing jitter between toggles due to electronic shot noise in the circuit, a source of entropy is introduced.

# Backend Circuitry

## FIFO Buffer

The 8-bit FIFO buffer mixes and caches the 2 incoming sources of entropy, from both the Geiger counter and the neoTRNG, and releases the data to the randomness extractor module once it has stored 32 bytes (512 bits). This ensures a fair mix of incoming data and steady data input into the next stage.

## Randomness Extractor

Given the source of radiation, the environment, is constant, there is a high likelihood that the data retrieved is biased towards a certain range of numbers. To remove the potential bias, a randomness extractor is used, in the form of a BLAKE2s hash function, like how Linux kernel RNG uses BLAKE2s as its entropy extractor [3]. The BLAKE2s hash function mixes and scrambles the input bits in such a way that the output doesn’t reflect the pattern. Additionally, this ensures that the TRNG system is preimage resistant and collision resistant, generating outputs that are uniformly distributed and free of observable bias.

## Serial UART Interface

The PRNG output is transmitted over a Universal Asynchronous Receiver-Transmitter (UART) interface for external use. This module is equipped with its own FIFO to accommodate bursty data transmission. The output is then logged and stored in a text file.

# Validation and Tesing

## Debugging

Debugging is conducted through the Xilinx Integrated Logic Analyzer (ILA), which greatly reduced debugging time and allowed us to identify issues rapidly.

Debugging with Vivado Integrated Logic Analyzer

## Quality Testing

The NIST Statistical Test Suite (based on the NIST Special Publication 800-22 specification) [5] was used to evaluate whether the outputs from the number generator is suitable for cryptographic grade randomness. The test suite includes tests such as Frequency Test, Approximate Entropy Test, Maurer’s “Universal Statistical” Test.

The input parameters for the test are:

Length of tested bitstream: 1,600,000 bits

Number of bitstreams: 50

Tested binary file size: 1,600,000 bits×50=10MByte

The results obtained were all above the recommended threshold of 47/50 and 33/36 for statistical tests and random excursion tests respectively. For the full results, please refer to Appendix A.

# Conclusion

In conclusion, this project presents a practical and cost-effective approach to generating cryptographically secure random numbers by leveraging both environmental radiation and electronic jitter as entropy sources. Through the integration of a Geiger counter and a ring oscillator-based neoTRNG, the system produces high-entropy, bias-free random outputs suitable for cryptographic applications.

The implementation on an FPGA not only ensures hardware-level security but also demonstrates the feasibility of building secure random number generators with accessible components. Validation through the NIST statistical test suite further confirms the reliability and effectiveness of the design.

##### Acknowledgment

We would like to express our heartfelt gratitude to all those who have guided and supported us throughout the course of this project.

First and foremost, we extend our deepest appreciation to Dr. Teo Tee Hui, our mentor from SUTD, for his invaluable insights, guidance, and encouragement at every stage of this project. His expertise and constructive feedback have been instrumental in shaping our work.

##### References

G. Bonacini, Nuclear RNG Generator, GitHub Repository: https://github.com/gbonacini/nuclear_rng_generation2

Herrero-Collantes, M., & Garcia-Escartin, J. C. (2017). Quantum random number generators. Reviews of Modern Physics, 89(1). https://doi.org/10.1103/revmodphys.89.015004

Saarinen, M. O., & Aumasson, J. (2015). The BLAKE2 Cryptographic Hash and Message Authentication Code (MAC). RFC, 7693, 1–30. https://doi.org/10.17487/RFC7693

Nir, Y., & Langley, A. (2015). CHACHA20 and Poly1305 for IETF protocols. https://doi.org/10.17487/rfc7539

Rukhin, A., Sota, J., Nechvatal, J., Smid, M., Barker, E., Leigh, S., Levenson, M., Vangel, M., Banks, D., Heckert, A., Dray, J., & Vo, S. (2000). A statistical test suite for random and pseudorandom number generators for cryptographic applications. https://doi.org/10.6028/nist.sp.800-22

##### Appendix A

##### NIST Test Suite Run Results

------------------------------------------------------------------------------

RESULTS FOR THE UNIFORMITY OF P-VALUES AND THE PROPORTION OF PASSING SEQUENCES

------------------------------------------------------------------------------

generator is <serial_dump.bin>

------------------------------------------------------------------------------

C1  C2  C3  C4  C5  C6  C7  C8  C9 C10  P-VALUE  PROPORTION  STATISTICAL TEST

------------------------------------------------------------------------------

2   5   5   7   3   5   4   7   8   4  0.699313     50/50      Frequency

3   6   5   6   3   2   5   8   6   6  0.739918     50/50      BlockFrequency

5   1   7   5   5   5   6   7   2   7  0.574903     49/50      CumulativeSums

3   4   7   8   7   5   2   6   5   3  0.616305     50/50      CumulativeSums

2   7   3   2   5   6   7   7   4   7  0.534146     50/50      Runs

5   5   4   4   3   4   7   8   9   1  0.319084     48/50      LongestRun

4  11   6   6   5   0   5   4   5   4  0.153763     49/50      Rank

7   8  10   3   6   2   3   3   5   3  0.171867     48/50      FFT

7   5   2   6   7   1   3   7   7   5  0.419021     50/50      NonOverlappingTemplate

4   4   3   6   4   6   6   5   9   3  0.739918     50/50      NonOverlappingTemplate

5   7  10   2   3   3   2   8   5   5  0.171867     49/50      NonOverlappingTemplate

3   6   8   3   5   7   4   3   4   7  0.699313     50/50      NonOverlappingTemplate

11   4   3   5   7   5   3   7   0   5  0.075719     50/50      NonOverlappingTemplate

10   4   5   4   7   3   3   5   4   5  0.534146     49/50      NonOverlappingTemplate

4   7   1   2  11   3   6   7   5   4  0.085587     49/50      NonOverlappingTemplate

11   5   2   3   2   6   2   7   7   5  0.085587     48/50      NonOverlappingTemplate

5   5   6   3   2   4   6   5   9   5  0.699313     50/50      NonOverlappingTemplate

6   3   5   8   6   8   3   3   5   3  0.616305     50/50      NonOverlappingTemplate

5   6   3   5   6   4  10   3   4   4  0.574903     50/50      NonOverlappingTemplate

8   4  11   5   2   6   5   3   2   4  0.122325     49/50      NonOverlappingTemplate

7   4   4   3   2   6   5   2  10   7  0.236810     49/50      NonOverlappingTemplate

5   6   6   4   8   3   9   2   3   4  0.419021     50/50      NonOverlappingTemplate

5   8   6   4   5   3   7   3   8   1  0.383827     49/50      NonOverlappingTemplate

6   5   8   2   3   4   6   7   5   4  0.739918     49/50      NonOverlappingTemplate

4   3   3   5  10   2   7   6   5   5  0.383827     50/50      NonOverlappingTemplate

5   7   4   1   3   4   6   9   3   8  0.262249     50/50      NonOverlappingTemplate

4   4   8   6   6   2   4   5   4   7  0.779188     50/50      NonOverlappingTemplate

4   8   1   4   5  10   5   5   3   5  0.262249     49/50      NonOverlappingTemplate

7   3   4  10   3   4   7   1   4   7  0.171867     49/50      NonOverlappingTemplate

5   5   5   5   4   4   4   4   9   5  0.911413     50/50      NonOverlappingTemplate

5   5   4   6   7   3   6   7   2   5  0.851383     48/50      NonOverlappingTemplate

6   6   2   5   4   6   6   5   4   6  0.955835     50/50      NonOverlappingTemplate

5   8   3   5   3   6   6   5   6   3  0.851383     49/50      NonOverlappingTemplate

6   5   3   3   5   8   5   5   3   7  0.816537     50/50      NonOverlappingTemplate

2   6   7   6   3   5   5   4   7   5  0.851383     50/50      NonOverlappingTemplate

5   4   3   8   3   2   3   8   6   8  0.350485     50/50      NonOverlappingTemplate

4   3   8   4   7   2   6   3   7   6  0.574903     50/50      NonOverlappingTemplate

7   5   4   5   5   6   4   5   5   4  0.996335     49/50      NonOverlappingTemplate

7   6   5   3   5   5   3   5   3   8  0.816537     49/50      NonOverlappingTemplate

8   1   4   9   4   6   7   6   1   4  0.153763     50/50      NonOverlappingTemplate

2   7   2   4   6   5   6   6   7   5  0.739918     50/50      NonOverlappingTemplate

6   9   5   6   6   5   4   1   5   3  0.534146     50/50      NonOverlappingTemplate

10   5   5   4   4   5   4   5   5   3  0.699313     50/50      NonOverlappingTemplate

6   7   3   6   0   6   4   3   6   9  0.236810     49/50      NonOverlappingTemplate

2   3   6   6   6   1   8   8   7   3  0.236810     50/50      NonOverlappingTemplate

6   9   6   2   5   3   6   3   6   4  0.574903     50/50      NonOverlappingTemplate

2   8   4   3   3   7   6   4   4   9  0.350485     50/50      NonOverlappingTemplate

4   2   3  10   3   7   2   8   5   6  0.153763     49/50      NonOverlappingTemplate

4   2   4   6   6   4  10   5   4   5  0.534146     50/50      NonOverlappingTemplate

6   2   8   6   7   6   1   3   5   6  0.419021     48/50      NonOverlappingTemplate

4   4   4   5   7   6   5   4   4   7  0.971699     50/50      NonOverlappingTemplate

4   8   5   5   4   7   1  11   3   2  0.066882     49/50      NonOverlappingTemplate

5   3   7   5   7   3   5   5   6   4  0.935716     50/50      NonOverlappingTemplate

6   7   5   7   4   4   6   1   6   4  0.739918     50/50      NonOverlappingTemplate

6   5   3   4   6   4   3   8   5   6  0.883171     50/50      NonOverlappingTemplate

8   3   1   5   5   6   7   5   4   6  0.616305     49/50      NonOverlappingTemplate

7   2   4   6   8   2   6   4   8   3  0.383827     50/50      NonOverlappingTemplate

3   4  11   2   3   3   9   5   7   3  0.058984     50/50      NonOverlappingTemplate

3   1   5   5   6   5   9  10   3   3  0.122325     50/50      NonOverlappingTemplate

8   7   3   8   3   6   3   1   5   6  0.319084     48/50      NonOverlappingTemplate

7   7   6   5   2   4   6   2   8   3  0.494392     49/50      NonOverlappingTemplate

7   2   7   7   7   1   4   6   7   2  0.262249     48/50      NonOverlappingTemplate

8   8   6   6   3   3   5   2   5   4  0.574903     49/50      NonOverlappingTemplate

5   5   6   4   2   6   0  10   9   3  0.058984     49/50      NonOverlappingTemplate

5   2   3   4   6   6   9   4   6   5  0.657933     49/50      NonOverlappingTemplate

6   2   7   3   8   7   5   6   2   4  0.494392     50/50      NonOverlappingTemplate

5   4   7   2   5   6   4   5   8   4  0.816537     49/50      NonOverlappingTemplate

4  11   7   7   2   4   1   3   5   6  0.085587     50/50      NonOverlappingTemplate

6   5   6   2   3   7   8   6   5   2  0.574903     49/50      NonOverlappingTemplate

2   0   6   7   4   5   5   6   7   8  0.289667     50/50      NonOverlappingTemplate

9   6   2   4   6   3   2   8   5   5  0.350485     49/50      NonOverlappingTemplate

5   6   5   5   8   1   8   4   6   2  0.419021     50/50      NonOverlappingTemplate

3   5   7   4   5   4   6   7   6   3  0.911413     50/50      NonOverlappingTemplate

8   1   3   7   6   4   2   7   7   5  0.319084     49/50      NonOverlappingTemplate

7   9   5   3   4   7   3   2   6   4  0.455937     50/50      NonOverlappingTemplate

9   5   2   5   2   4   3   6   3  11  0.066882     50/50      NonOverlappingTemplate

3   8   6   5   5   4   5   7   5   2  0.779188     48/50      NonOverlappingTemplate

5   2   1   4   4  10   6   4   4  10  0.066882     50/50      NonOverlappingTemplate

4   8   2   6   6   2   5   9   2   6  0.262249     49/50      NonOverlappingTemplate

7   1   2   8   4   6   8   2   4   8  0.137282     50/50      NonOverlappingTemplate

5   7   2   3   6   4   3   6   8   6  0.657933     49/50      NonOverlappingTemplate

6   4   9   4   2   2   4   7   4   8  0.319084     50/50      NonOverlappingTemplate

7   5   2   6   7   1   3   7   7   5  0.419021     50/50      NonOverlappingTemplate

8   4   5   8   5   4   4   2   5   5  0.739918     49/50      NonOverlappingTemplate

7   6   5   8   5   2   3   5   5   4  0.779188     50/50      NonOverlappingTemplate

3   3   3   6   5   6   6   2   6  10  0.350485     50/50      NonOverlappingTemplate

4   3   5   6   9   5   2   4   6   6  0.657933     50/50      NonOverlappingTemplate

5   1   5   4   7   3   6   8   6   5  0.616305     50/50      NonOverlappingTemplate

2   6   4   2   2   7   9   5   4   9  0.153763     50/50      NonOverlappingTemplate

5   4   7   7   5   1   7   5   6   3  0.657933     50/50      NonOverlappingTemplate

6   4   5   2   7   7   5   6   4   4  0.883171     49/50      NonOverlappingTemplate

7  11   3   3   4   9   1   6   2   4  0.030806     50/50      NonOverlappingTemplate

9   4   2   4   8   4   7   2   7   3  0.236810     50/50      NonOverlappingTemplate

10   2   6   2   6   1   5   6   8   4  0.108791     50/50      NonOverlappingTemplate

12   2   6   6   6   6   5   2   3   2  0.051942     49/50      NonOverlappingTemplate

8   5   4   2   8   3   5   8   4   3  0.419021     50/50      NonOverlappingTemplate

6   5   2   9   4   3   8   3   6   4  0.419021     49/50      NonOverlappingTemplate

8   3   3   3   4   3   7   6   9   4  0.383827     49/50      NonOverlappingTemplate

5   8   4   4   4   6   3   8   3   5  0.739918     49/50      NonOverlappingTemplate

3   4   5   4   6   9   3   5   8   3  0.534146     50/50      NonOverlappingTemplate

2   6   7   8   5   6   3   7   5   1  0.383827     50/50      NonOverlappingTemplate

6   5   5   3   7   7   4   5   3   5  0.935716     50/50      NonOverlappingTemplate

8   4   7   1   5   2   6   7   3   7  0.319084     48/50      NonOverlappingTemplate

3   6   2   5   4   7   6   7   4   6  0.816537     49/50      NonOverlappingTemplate

2   8   6   6   5   3   6   5   5   4  0.816537     50/50      NonOverlappingTemplate

8   9   4   4   7   1   5   3   5   4  0.319084     49/50      NonOverlappingTemplate

6   6   4   2   3   5   5   5   8   6  0.816537     49/50      NonOverlappingTemplate

6   7   5   5   3   5   5   2   5   7  0.883171     50/50      NonOverlappingTemplate

4   7   7   2   3   5   6   5   6   5  0.851383     50/50      NonOverlappingTemplate

6   3   8   2   7   6   4   4   6   4  0.699313     50/50      NonOverlappingTemplate

6   4   5   6   6   2   4   2   7   8  0.616305     50/50      NonOverlappingTemplate

4   2   4  10   6   5   6   5   4   4  0.534146     50/50      NonOverlappingTemplate

5   6   6   8   7   2   3   2   7   4  0.494392     50/50      NonOverlappingTemplate

9   1   5   5   4   6   5   4   5   6  0.616305     50/50      NonOverlappingTemplate

2   4   5   5   8   4   9   4   5   4  0.574903     50/50      NonOverlappingTemplate

8   4   3   4   8   4   6   3   6   4  0.699313     49/50      NonOverlappingTemplate

1   9   4   6   5   5   5   5   2   8  0.319084     50/50      NonOverlappingTemplate

6   6   8   6   4   7   6   3   2   2  0.534146     50/50      NonOverlappingTemplate

6   6   5   2   7   4   3   8   4   5  0.739918     47/50      NonOverlappingTemplate

5   5   5   5   5   6   7   3   5   4  0.991468     50/50      NonOverlappingTemplate

5   3   8   3   3   8   7   3   4   6  0.534146     49/50      NonOverlappingTemplate

3   1   5   3   7   7   3   8   6   7  0.350485     50/50      NonOverlappingTemplate

4   6   6   2   5   1   6   9   8   3  0.236810     50/50      NonOverlappingTemplate

2   1   4  10   3  10   4   7   6   3  0.035174     50/50      NonOverlappingTemplate

5   4   6   5   7   1   7   3   4   8  0.534146     50/50      NonOverlappingTemplate

8   3   6   3   2  10   4   5   4   5  0.289667     50/50      NonOverlappingTemplate

5   6   4   7   7   2   3   3   8   5  0.616305     50/50      NonOverlappingTemplate

5   2   5  10   4   5   1   9   6   3  0.108791     50/50      NonOverlappingTemplate

5   3   2   7   3   6   5   6   4   9  0.534146     49/50      NonOverlappingTemplate

6   9   7   4   5   3   3   4   6   3  0.616305     50/50      NonOverlappingTemplate

5   9   4   8   4   2   7   6   1   4  0.236810     50/50      NonOverlappingTemplate

4   4   6   8   4   2   6   6   4   6  0.816537     49/50      NonOverlappingTemplate

5  12   3   4   5   4   4   2   5   6  0.153763     50/50      NonOverlappingTemplate

4   6   6   4   3   6   5   5   7   4  0.971699     50/50      NonOverlappingTemplate

1   4   3   8   5   8   6   8   4   3  0.289667     50/50      NonOverlappingTemplate

1   7   3   7   5   7   3   4   7   6  0.494392     50/50      NonOverlappingTemplate

3   2   6   9   5   5   4   4   5   7  0.616305     48/50      NonOverlappingTemplate

3   5   6   6   7   8   5   2   6   2  0.574903     50/50      NonOverlappingTemplate

4   5   8   4   8   4   5   4   4   4  0.851383     49/50      NonOverlappingTemplate

6   4   5   4   3   5   5   9   5   4  0.851383     50/50      NonOverlappingTemplate

4   4   8   6   4   3   5   4   8   4  0.779188     50/50      NonOverlappingTemplate

4   8   4   6   8   0   2   4   5   9  0.108791     50/50      NonOverlappingTemplate

6   6   2   9   4   6   5   4   3   5  0.657933     50/50      NonOverlappingTemplate

8   6   2   4   3   8   6   6   5   2  0.455937     50/50      NonOverlappingTemplate

4   5   3   4   6   6   3   3   8   8  0.657933     50/50      NonOverlappingTemplate

6   5   4   4   1   4   4   6   8   8  0.534146     49/50      NonOverlappingTemplate

3   5   9   3   2   7   2   5  10   4  0.108791     50/50      NonOverlappingTemplate

3   6   7   5   2   5   8   2   5   7  0.534146     50/50      NonOverlappingTemplate

5   3   3   3   8   3  10   5   6   4  0.319084     50/50      NonOverlappingTemplate

6   4   7   5   3   9   7   4   3   2  0.455937     49/50      NonOverlappingTemplate

1   7   5   7   4   4   8   3   6   5  0.534146     50/50      NonOverlappingTemplate

5   5   4   4   8   5   3   6   6   4  0.935716     49/50      NonOverlappingTemplate

5   4   6   5   9   7   3   3   6   2  0.534146     50/50      NonOverlappingTemplate

3   6   7   6   8   5   5   4   3   3  0.779188     50/50      NonOverlappingTemplate

3   7   4   5   7   6   3   8   4   3  0.699313     50/50      NonOverlappingTemplate

6   4   9   4   2   2   4   7   4   8  0.319084     50/50      NonOverlappingTemplate

4   6   8   4   4   6   3   4   4   7  0.851383     50/50      OverlappingTemplate

6   4   8   2   6   5   2   4   7   6  0.616305     49/50      Universal

4   3   8   6   4   8   5   5   2   5  0.657933     50/50      ApproximateEntropy

5   6   3   3   2   5   4   4   1   3  0.534146     35/36      RandomExcursions

5   9   2   1   3   4   4   2   4   2  0.054199     34/36      RandomExcursions

5   2   6   5   0   5   5   0   5   3  0.066882     36/36      RandomExcursions

5   3   6   2   4   4   6   1   2   3  0.350485     36/36      RandomExcursions

4   3   2   3   2   4   5   7   1   5  0.299251     36/36      RandomExcursions

3   2   3   5   5   4   3   4   3   4  0.911413     36/36      RandomExcursions

4   2   4   4   4   4   5   4   3   2  0.911413     36/36      RandomExcursions

2   3   0   2   3   3   7   3  10   3  0.002624     35/36      RandomExcursions

4   3   6   3   3   8   2   4   1   2  0.122325     36/36      RandomExcursionsVariant

3   2   7   6   1   5   2   6   3   1  0.066882     36/36      RandomExcursionsVariant

4   2   9   3   2   2   2   3   5   4  0.082177     36/36      RandomExcursionsVariant

3   4   6   5   3   1   5   3   1   5  0.350485     36/36      RandomExcursionsVariant

3   4   7   5   6   2   1   3   3   2  0.213309     35/36      RandomExcursionsVariant

3   6   7   3   4   3   2   2   3   3  0.407091     35/36      RandomExcursionsVariant

6   3   3   2   5   4   8   3   0   2  0.054199     35/36      RandomExcursionsVariant

4   9   0   1   4   5   5   4   2   2  0.014216     35/36      RandomExcursionsVariant

4   5   3   4   5   3   0   8   0   4  0.035174     35/36      RandomExcursionsVariant

2   4   0   6   4   6   6   5   1   2  0.066882     36/36      RandomExcursionsVariant

2   3   5   5   3   2   3   7   4   2  0.407091     36/36      RandomExcursionsVariant

4   4   4   2   2   2   2   5   4   7  0.407091     35/36      RandomExcursionsVariant

4   3   2   3   2   1   2   6   6   7  0.122325     36/36      RandomExcursionsVariant

5   1   1   3   3   2   3   3   5  10  0.008879     35/36      RandomExcursionsVariant

3   4   4   1   1   5   2   5   4   7  0.213309     35/36      RandomExcursionsVariant

5   4   0   4   3   7   2   1   4   6  0.082177     35/36      RandomExcursionsVariant

6   3   2   3   2   5   4   4   3   4  0.739918     35/36      RandomExcursionsVariant

6   5   3   2   4   3   1   4   2   6  0.350485     34/36      RandomExcursionsVariant

5   5   4   4   6   5   7   6   4   4  0.991468     50/50      Serial

1  10   6   1   8   5   6   6   5   2  0.075719     50/50      Serial

9   8   2   4   5   5   5   3   6   3  0.455937     50/50      LinearComplexity

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

The minimum pass rate for each statistical test with the exception of the

random excursion (variant) test is approximately = 47 for a

sample size = 50 binary sequences.

The minimum pass rate for the random excursion (variant) test

is approximately = 33 for a sample size = 36 binary sequences.

For further guidelines construct a probability table using the MAPLE program

provided in the addendum section of the documentation.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -